Software-Defined Computer Design Document

Project Name: Software-Defined CPU

Author: Blake Minix

Date: 12-18-2025

- CPU Overview

Type: 8-register, 64KB memory, software-defined CPU

Registers: 8 general-purpose (R0-R7), plus PC and SP

Memory: 64KB array

Stack: LIFO, grows down from top of memory

Word size: 32-bit registers, 8-bit memory cells (can adjust later)

Endianness: Little-endian (for multi-byte values)

I/O: Console output (PRINT)

Purpose: Minimal CPU capable of running small programs in software

Diagram (simplified):

[Registers] <-> [ALU] <-> [Memory/Stack]

|

v

[I/O]

- Registers

R0-R7: General-purpose registers

PC: Program counter (points to next instruction)

SP: Stack pointer (initially top of memory, stack grows down)

Z: Zero flag (set when arithmetic result = 0)

- Memory

Total size: 64KB

Instruction fetch reads memory as bytes; multi-byte operands are read sequentially

Layout:

0x0000 - 0x7FFF: Code segment

0x8000 - 0xBFFF: Heap (optional)

0xC000 - 0xFFFF: Stack

Access: All memory readable/writable; instructions will enforce boundaries

- Stack

Type: LIFO

Operations: Planned (PUSH, POP, CALL, RET)

Growth: downwards from 0xFFFF

Purpose: Temporary storage, function calls

- Instruction Set (minimal)

0x01: ADD: Rdest, Rsrc1, Rsrc2: Rdest = Rsrc1 + Rsrc2: ADD R2, R0, R1

0x02: SUB: Rdest, Rsrc1, Rsrc2: Rdest = Rsrc1 - Rsrc2: SUB R3, R2, R1

0x03: MOV: Rdest, Immediate: Load Immediate into register: MOV R0, 5

0x04: PRINT: Rsrc: Print value of register: PRINT R2

0x05: LOAD: Rdest, Address: Load memory[Address] -> Rdest: LOAD R0, 0x1000

0x06: STORE: Rsrc, Address: Store Rsrc -> memory[Address]: STORE R1, 0x1000

0x07: JMP: Address: PC = Address: JMP 0x000A

0x08: JZ: Address: Jump if Z flag set: JZ 0x0010

0x09: JNZ Address: Jump if Z flag not set: JNZ 0x0020

0xFF: HALT: - : Stop execution: HALT

Instructions are byte-encoded in memory.

Example encodings:

ADD R2, R0, R1 -> [opcode][dest][src1][src2]

MOV R0, 5 -> [opcode][dest][imm8]

JMP 0x0010 -> [opcode][addr_lo][addr_hi]

Immediates are 8-bit values (zero-extended to 32-bit registers)

The Z flag is updated by arithmetic and MOV instructions.

Instruction length is determined by opcode and operand count.

- Program Execution (Fetch-Decode-Execute)

Loop:

Fetch instruction from memory at PC

Decode opcode and operands

Execute instruction

Update PC

Repeat until HALT

- I/O

PRINT Rsrc: prints value of register to console

Optional: memory-mapped framebuffer for 2D graphics

Input (keyboard/mouse) optional

- Optional/Future Extensions

Stack operations (PUSH, POP, CALL, RET)

Status flags: Zero, Negative, Overflow

Virtual memory or memory protection

Multitasking/preemptive scheduler

Graphics framebuffer for games

JIT compilation for speed

Debugger/visualizer

Development Path:

- Implement CPU struct (registers, memory, PC, SP)

- Implement ADD, SUB, MOV, PRINT, HALT

- Write fetch-decode-execute loop

- Run tiny test programs

- Incrementally add LOAD/STORE, JMP, stack ops

- Optional: Assembler, I/O, graphics, games